Document Version: 2001.001

Dear SPRINT user,

when you get the screen message "Please ask your distributor for MODxxx..., a hardware change is necessary for that special device you want to program. The required change is easy to do, however there are 120 pins that must be carefully separated and reconnected. Since these pins can be bent, please exercise appropriate care in disassembling and reassembling the boards.. If you have any problems don't hesitate to call SMS or send us the TOP.

This modification is concerning some devices who needs a fast pull-up on a special pin or who have the supply-pins not at the standard locations. Look to the schematic on the page 2 of this document to locate the positions of the pins and the requested solder pads.

1)	Remove the TOP1PLC of your programmer.

2)	Remove the four screws fixing the PCB in the TOP (not in the programmer).

3)	The TOP1PLC consists of two PCBs. Detach them carefully and take out only the first one. The second PCB should remain at its position. The hardware change will be performed on the component side of the PCB.

Note :	Vx, Gx and Px means select one of the spare relais V1, V2 (G1, G2) 
	or spare clocks (P1, P10)

Attach the two PCB's together carefully. Put them back into the TOP and screw in the four screws. Start the PAL software and select the device to check your work. The message "Please ask your distributor...." should no longer appear on the screen.

WARNING : It is not posible to have two different modifications in the same TOP, that are using the same signals (e.g. V1, G1 etc). Only modifications that are using different signals can be installed in one TOP1PLC.  Examples : MOD5/93 and MOD19/94 in one TOP are not possible. MOD8/93 and MOD2/94 in one TOP are possible !

| MOD | Device | Signal | Pin-number |
| --- | --- | --- | --- |
| 5/93 | Altera 'old' EPM7128-84, 'old' EPM7160-84 | connect both : Fast Pull-up P1 Fast pull-up P10 | 74 77 |
| 8/93 | Intel iFX780 | Fast pull-up P10 GND G1 | 53 17 |
| 2/94 | Cypress 7C37x | VCC V1 | 42 |
| 4/94 | Intel iFX740-44/68 | GND G1 GND G2 | 66 67 |
| 13/94 | AMD MACH131 | Fast pull-up P10 | 38 |
| 19/94 | ALTERA EPM 7128E/7160E -84 "new" 7128/7160-84 | connect both : Fast pull-up P1 Fast pull-up P10 | 70 11 |
| 2/95 | ALTERA EPM7064-44 | connect Fast pull up Px | 80 |
| 17/95 | CYPRESS Palce16V8 | connect Fast pull up Px | 22 |
| 6/96 | Philips ABT22V10A LVT22V10 | solder a 10 K resistor between V1 and Pin 67 and between V2 and Pin 82 |  |
| 10/97 | Atmel ATF22V10B/C | connect Fast pull up Px | 40 |

Pin- and Solderpad Positions

Component side view of the upper PCB